<INSERT_TITLE_START> Phase 2 Environmental Site Assessment for the 09-02-014-03 W4M Wellsite <INSERT_TITLE_END>

<INSERT_CLIENT_START> Prepared For: Canlin Energy Corporation 237 4 Ave SW Calgary, AB T2P 4K3 <INSERT_CLIENT_END>

Prepared By: EBM Geoscience Inc. 204, 1135 - 64 Ave SE Calgary, AB T2H 2J7

<INSERT_DATE_START> Date: Month XX, 2025 <INSERT_DATE_END>

Executive Summary

<INSERT_EXEC_SUMMARY_START>

Introduction

Facility

The CLIENT NAME (CLIENT ACRONYM) retained EBM Geoscience Inc. (EBM) to complete a Phase 2 Environmental Site Assessment (ESA) at the Questfire F30335 compressor station at surface location 15-14-014-04 W4M (the site). The purpose of the assessment was to investigate each area of potential environmental concern (APEC) for the appropriate contaminants of potential concern (CoPCs) and to confirm the presence or absence of contamination.

OR – for a wellsite

Canlin Energy Corporation (Canlin) retained EBM Geoscience Inc. (EBM) to complete a Phase 2 Environmental Site Assessment (ESA) at the Canlin MEDHAT 9-2-14-3 wellsite at surface location 100/09-02-014-03W4M (the site). The purpose of the assessment was to investigate each area of potential environmental concern (APEC) for the appropriate contaminants of potential concern (CoPC) and to confirm the presence or absence of contamination.

History + Purpose

Default

Previous assessments included a Phase 1 ESA, a geophysical investigation, and a soil investigation. The identified APECs included the well centre, the drilling waste disposal area (DWDA), and an electromagnetic (EM) anomaly. The CoPCs included chloride-related salinity impact, petroleum hydrocarbons (PHCs), polycyclic aromatic hydrocarbons (PAHs), glycols, and metals.

Recent Investigation Summary + Chloride Delineation

Default – first investigation on-site

The soil investigation was conducted from November 22–23, 2024. Eight boreholes were advanced to a maximum depth of 9.0 m. Vertical and lateral delineation to the Tier 1 standard of 100 mg/L chloride was achieved

OR – Justification lateral delineation, previous investigations conducted

The recent soil investigation was conducted on November 22, 2024. Eight boreholes were advanced to a maximum depth of 9.0 m. Vertical and lateral delineation to the Tier 1 standard of 100 mg/L chloride was achieved, except for lateral delineation to the east. However, lateral delineation was inferred based on the decreasing concentrations and the EM survey.

OR - Multiple recent investigations and delineation not achieved

The recent soil investigations were conducted on November 22, 2024 and from April 18-19, 2025. Twenty boreholes were advanced to a maximum depth of 15.0 m. Vertical delineation to the Tier 1 standard of 100 mg/L chloride was not achieved at various locations, and lateral delineation was not achieved to the east and west.

Or - Background chloride – first investigation on-site

The soil investigation was conducted on November 17, 2025. A total of 11 boreholes were advanced to a maximum depth of 6.0 m. Elevated background chloride was identified at two locations, with a maximum concentration of 150 mg/L. Vertical and lateral chloride delineation to the Tier 1 standard of 100 mg/L or to background levels was achieved. Section 5 provides a site-specific evaluation and justification of the background chloride.

Exceedance Summary + Conclusion

Default: no exceedances identified, no further work required

Soil concentrations and salinity parameters at all sampled locations were below the applicable Tier 1 guidelines. Based on these results, no further remedial action is required.

OR – No exceedances, but delineation required

Soil concentrations and salinity parameters at all sampled locations were below the applicable Tier 1 guidelines. Based on these results, further investigation may be warranted to complete the chloride delineation.

OR – Exceedances, but delineation required

Salinity exceedances were identified at the well centre and EM anomaly, while the remaining soil concentrations and salinity parameters complied with the applicable Tier 1 guidelines. Based on these results, further remediation action is required.

OR – soil exceedances, no volume, further work is required

Salinity exceedances were identified at the well centre area, while salinity, hydrocarbon, and metal exceedances were identified at the DWDA. No exceedances were identified at the EM anomaly. Based on these results, further remediation action is required.

Or - -justifications – further work not ‘recommended’

Salinity and metal exceedances were identified in the DWDA. The salinity exceedances are associated with high sulphate and low chloride concentrations and were assumed to be naturally occurring. The metal exceedance is marginal, isolated, and assumed to be naturally occurring and of low risk.

Based on these results, no further remedial action is recommended.

Or – exceedances noted, volume estimated, further work required

Soil exceedances were not identified at the well centre or EM anomaly. However, salinity and hydrocarbon exceedances were identified at the DWDA. The estimated impact volume is 400 m3 (720 tonnes).

Based on these results, further remediation action is required.

OR  - Lots of exceedances, one justification, and a volume estimate

No soil exceedances were identified at the riser system, heater tank, separator, or flare stack. At the former infrastructure area, a marginal exceedance justification was applied to a salinity exceedance. At the compressor building, salinity and PHC exceedances were identified, with an estimated volume of 345 m3 (621 tonnes).

Based on these results, further remediation action is required.

<INSERT_EXEC_SUMMARY_END>

Table of Contents

1	Introduction	1

1.1	Objectives and Scope	1

1.2	Background Information	1

1.2.1	Previous Environmental Assessments	1

1.2.2	Preliminary Work and Field Activities	2

1.2.3	Regulatory Requirements	2

2	Site Characterization	3

2.1	Facility Information	3

2.2	Stakeholders	3

2.3	Conceptual Site Model	4

2.3.1	Site Characteristics and Contaminant Sources	4

2.3.2	Regional Information	4

2.3.3	Receptor Distances	6

2.3.4	Soil Characteristics	6

2.3.5	Groundwater Characteristics	6

2.3.6	Exposure Pathway Evaluation	6

3	Guideline Selection	9

3.1	Tier 1	9

3.2	SCARG	9

3.3	Tier 1 with Background	10

4	Soil Assessment Results	11

4.1	Borehole Rationale	11

4.2	Site Characterization	12

4.2.1	Background	12

4.2.2	Well Centre - Clean	13

4.2.3	Well Centre - Impacted	14

4.2.4	Well Centre – Justified EC	14

4.2.5	Well Centre – Justified SAR	16

4.2.6	Drilling Waste Disposal Area	16

4.2.7	Drilling Waste Disposal Area – NIL FINDINGS	17

4.2.8	Separator/Methanol Tank	18

4.2.9	Infrastructure #2	18

4.2.10	Infrastructure #3	19

4.2.11	Surface Staining	19

4.3	Tier 1 Chloride Delineation	19

5	Background Chloride	22

5.1	Historical Site Activities	22

5.2	Contaminant Transport Mechanisms	23

5.3	Chemistry Footprint	23

5.4	Background Chloride Summary	25

6	Impact Volume	26

7	Compliance Summary	28

7.1	APEC Review	28

7.2	Justification Summary	28

8	Summary and Conclusions	32

9	Closure	34

10	Limitations of the Report	36

11	References	38

# Figures

Figure 1: Soil Chloride Profile

Figure 2: Chemistry Footprint: Potassium vs. Chloride

Figure 3: Chemistry Footprint: Sulphate vs. Chloride

Figure 4: Site Location Map

Figure 5: Soil Exceedances Assessment

Or

Figure 1: Soil Chloride Profile

Figure 2: Site Location Map

Figure 3: Soil Salinity and pH Assessment

Figure 4: Soil Hydrocarbon Assessment

Figure 5: Soil Metal Assessment

Figure 6: Residual Exceedances

# Tables

Table 1: SCARG Guidelines

Table 2: Tier 1 with Background Guidelines

Table 3: Borehole Rationale

Table 4: Remediation Volume Estimate

Table 5: APEC and COPC Investigation Summary

Table 6: Concordance Table

Table 7: Residual Exceedances

Table 6: Soil Salinity and Texture Analysis

Table 7: Soil Hydrocarbon Analysis

Table 8: Soil Metal Analysis

# Appendices

Appendix A: Borehole Logs

Appendix B: Laboratory Reports

Appendix C: Geophysical Surveys

Appendix D: Methodology

Appendix E: Background Statistics

Appendix F: Professional Declaration

# Introduction

<INSERT_1_START>

Canlin Energy Corporation (Canlin) retained EBM Geoscience Inc. (EBM) to complete a Phase 2 Environmental Site Assessment (ESA) at the Canlin MEDHAT 9-2-14-3 wellsite at surface location 100/09-02-014-03W4M (the site). Fieldwork was completed on November 19, 2025.

<INSERT_1_END>

## Objectives and Scope

<INSERT_1_1_START>

Presence/Absence P2

The objective of the assessment was to investigate each area of potential environmental concern (APEC) for the presence or absence of the contaminants of potential concern (CoPCs).

OR-Standard Tier 1 P1

The objectives of the assessment were to characterize soil quality at the identified areas of potential environmental concern (APECs), delineate the extent of impacts for the relevant contaminants of potential concern (CoPCs), and evaluate the site for closure.

The scope of work included the following:

Review the historical investigation data.

Evaluate background conditions.

Evaluate each APEC for the associated CoPCs.

Develop a conceptual site model (CSM).

Prepare a summary report to current regulatory standards.

- <INSERT_1_1_END>

## Background Information

### Previous Environmental Assessments

<INSERT_121_START>

The Phase 1 ESA was unavailable for review, and to our knowledge, no other contaminant investigations have been conducted.

OR

<INSERT_121_TABLE>

| 2021 (TLS, 2023) | The historical Phase 1 ESA made the following conclusions: The drilling mud additives met the compliance option requirements, and investigation of the drilling waste disposal area (DWDA) is not required. OR The drilling mud additives did not meet the compliance option requirements for salinity, and an investigation of the drilling waste disposal area (DWDA) is required. The well centre area requires investigation for potential environmental impacts associated with gas production and well-servicing activities. A visual inspection of the site should be conducted to identify additional areas of suspected impact. |
| --- | --- |
| 2024 (Matrix, 2024) | In October 2011, six boreholes were advanced to a maximum depth of 4.5 mbgs. EC and SAR exceedances with elevated chloride concentrations were identified northeast of the well center, to depths ranging from 2.0–4.0 m. Vertical delineation of the exceedances and soil chloride was not achieved. |
| 2026 (AKS, 2026) | One survey – no areas of high conductivity In June 2026, an electromagnetic (EM) 38 survey was conducted to help identify and delineate potential salinity impacts. No areas of significantly elevated apparent conductivity were identified. OR – one survey, one area of high conductivity In June 2026, an electromagnetic (EM) 31 survey was conducted to help identify and delineate potential salinity impacts. An area of elevated apparent conductivity was identified in the southwest portion of the lease. OR – two surveys, multiple areas of high conductivity In December 2022, electromagnetic (EM) 38 and 31 surveys were conducted to help identify and delineate potential salinity impacts. Areas of elevated apparent conductivity were identified to the west of the wellbore, in the southwest portion of the lease, and off-lease to the north and south. |

<INSERT_121_END>

### Preliminary Work and Field Activities

Prior to the fieldwork, the following preliminary activities were conducted:

As needed, all required crossing and proximity agreements were obtained from the third-party licensees.

- All ground disturbance protocols were executed in accordance with EBM’s standard operating procedures and the client’s specific requirements.

A sensitive species survey was completed across the project area. For sites with an identified sensitive species, a qualified wildlife monitor was present during equipment operations to minimize disruption.

Describe other preliminary activities, such as:

  - Approval/coordination with third-party consultants, i.e. work plan review with Lawson

  - Temporary access agreements

  - Specific third-party crossing or proximity agreements

  - Other landowner requirements, permissions, or agreements

  - Daylighting activities

# Conceptual Site Model

## Facility Information

<INSERT_WELLSITE_START>

Wellsite

<INSERT_WELLSITE_TABLE>

| Well Name | CNRL MEDHAT 14-22-14-2 |
| --- | --- |
| Legal Land Description | 100/00-00-000-00 W4 |
| Licensee | Canadian Natural Resources Limited |
| License No. | 0075496 |
| License Status | Active/Abandoned |
| Spud Date | June 5, 1979 |
| Abandonment Date | June 10, 1998 |
| Wellbore Depth | 563.0 m |
| Total gas production |  |
| Total water production |  |
| Total crude oil/bitumen production |  |
| Associated Spills | NA OR The Phase 1 ESA for the 100-well lists the following associated spills (Reference for Phase 1). Incident # 19973313. November 5, 1997. License #: 3608-2 (pipeline). Cause: Leak. Spill on-site. Substance spilled: 0.3 m3 of crude oil and 0.7 m3 salt/produced water, 0.3 m3 of crude oil and 0.6 m3 salt/produced water recovered. |

<INSERT_WELLSITE_END>

<INSERT_FACILITY_START>

Facility

<INSERT_FACILITY_TABLE>

| Site Name | AVN MEDICINE HAT 15-14-14-4W4 BTY & Compressor Station |
| --- | --- |
| Legal Land Description | 15-14-014-04 W4M |
| Licensee | Questfire Energy Corp. (Defunct) |
| License No. | F30335 |
| Facility Type | Gas multiwell production battery and compressor station |
| Construction Date | Exact date unknown, between 2003 and 2006 |
| Associated Spills | NA |

<INSERT_FACILITY_END>

## Stakeholders

| County/Municipality | Cypress County | Ownership Type | Freehold |
| --- | --- | --- | --- |
| Public Lands Type | NA | PLA Disposition | NA |
| Landowner / Occupant | Herman Land Cattle | Other Stakeholders | NA |

## Site Characteristics and Contaminant Sources

<INSERT_23_TABLE>

| Site Location | Approximately 35 km northeast of Medicine Hat (Figure XX). |
| --- | --- |
| Current Land Use | Agricultural. The site and surrounding areas are native prairie grassland. |
| Future Land Use |  |
| Adjacent Land Use More Sensitive | Yes / No |
| Current Infrastructure |  |
| APECs | Well Centre, flare pit, DWDA, EM anomalies |
| CoPCs | Salinity, PHC, metals |

## Regional Information

<INSERT_24_TABLE>

| Natural Subregion (Natural Regions Committee, 2006) | Dry Mixedgrass Subregion of the Grassland Natural Region. The largest and driest grassland subregion, located in southeastern Alberta. It features semi-arid prairie, coulees, badlands, and sand dunes, with sparse tree cover. Soils are mainly Solonetz and Brown Chernozems. Key species include pronghorn, sage grouse, burrowing owl, and prairie rattlesnake. Land use is dominated by grazing, dryland farming, and oil and gas development. Hot summers, drying winds, and low precipitation define its climate. Foothills Fescue Subregion of the Grassland Natural Region. The highest grassland Natural Subregion, with elevations between 800 – 1500 m. Foothills Fescue has the highest precipitation, warmest winters and shortest growing season of any grassland Subregion. Native communities are dominated by grasses, with little shrub or forest vegetation, compared to the Montane, Foothills Parkland, and Central Parkland Natural Subregions. Orthic Black Chernozems are the dominant soil type, with Dark Brown Chernozems occurring on southern slopes and wind-eroded areas near Pincher Creek. Water and Wetlands are rare, covering only 3% of the land area. Agriculture is the dominant land use, both cultivation and grazing. Proximity to the mountains significantly affects the region's climate, leading to higher precipitation and a greater frequency of chinooks. Northern Fescue Subregion of the Grassland Natural Region. A transition zone between grassland and parkland, with a mix of cultivated fields and rolling native prairie. Soils are Dark Brown Chernozems and Solonetz. Grasslands are moist and dominated by rough fescue, with aspen and shrubland patches in wetter areas. Wildlife includes yellow-headed blackbird, prairie crocus, and red-shanked grasshopper. 50% of the land is cultivated, with remaining areas used for grazing and oil and gas development. Central Parkland Subregion of the Parkland Natural Region. The most populated and intensively cultivated subregion in Alberta, forming a broad crescent from Calgary to Edmonton and east to Saskatchewan. It features a mosaic of aspen woodlands, shrublands, and grasslands. Soils are highly fertile, supporting wheat, barley, and canola crops. Wetlands cover about 10% of the area and serve as a major breeding ground for waterfowl. Wildlife includes the black-crowned night heron, Franklin’s ground squirrel, and piping plover. Dry Mixedwood Subregion of the Boreal Forest Natural Region. A transitional subregion between the Parkland and Boreal Forest, characterized by aspen-dominated mixedwood forests with balsam poplar, white spruce, and jack pine. It has a moderate climate with warm summers and cold winters. Soils are well-drained, supporting agriculture, forestry, and oil and gas development. Wetlands are scattered but provide key habitat for moose, black bears, and songbirds like the black-capped chickadee. |
| --- | --- |
| Landscape | Low-relief, hummocky. |
| Soil Types | The major soil types include well-drained Orthic Brown Chernozems developed from medium-textured till (Abacus, 2026). |
| Surface Geology | Colluvial Deposits. May contain pre-existing bedrock, till, glaciolacustrine, glaciofluvial and/or eolian sediments, generally poorly sorted (Abacus, 2026; EPA, 2024a). Eolian Deposits. Well-sorted, medium to fine-grained sand and minor silt. Includes both active and vegetated dunes, as well as sand sheets. Map label E (Abacus, 2026; EPA, 2024a). Fluvial Deposits. Poorly to well-sorted, stratified-to-massive sand, gravel, silt, clay and organic sediments. Map label F (Abacus, 2026; EPA, 2024a). Glaciofluvial Deposits. Sediments range from massive to stratified, poor to well-sorted, and coarse to fine-grained. Map label FG (Abacus, 2026; EPA, 2024a). Glaciers. Permanent snow and ice as of 1994. Ice with minor boulders to clay-sized sediment. Map Label G (Abacus, 2026; EPA, 2024a). Lacustrine Deposits. Includes offshore sand, silt, clay, and minor organic deposits; may also include nearshore beaches and bars composed of sand, silt, and minor gravel. Map label L (Abacus, 2026; EPA, 2024a). Glaciolacustrine Deposits. Massive fine sand, silt and clay or massive to stratified well-sorted sand. Map label LG (Abacus, 2026; EPA, 2024a). Moraine. Till, a mixture of clay, silt, sand, and minor pebbles, cobbles, and boulders. Map label M (Abacus, 2026; EPA, 2024a). Fluted Moraine. Sediments are mainly till and may locally include stratified glaciolacustrine and/or glaciofluvial sediments. Map label MF (Abacus, 2026; EPA, 2024a). Stagnant Ice Moraine. Sediments are mainly till but locally include stratified glaciolacustrine or glaciofluvial sediments. Map label MS (Abacus, 2026; EPA, 2024a). Ice-Thrust Moraine. Sediments may include syngenetic till, as well as masses of pre-existing sediments and/or bedrock. Map label MT (Abacus, 2026; EPA, 2024a). Organic Deposits. Woody to fibrous muck, commonly underlain by fine-grained, poorly drained glaciolacustrine or lacustrine deposits; in some places, underlain by till. Map label O (Abacus, 2026; EPA, 2024a). Bedrock. Map label R (Abacus, 2026; EPA, 2024a). Preglacial Fluvial Deposits. Predominantly well-sorted quartzite and chert gravel and cobbles with minor sand. Map label RT (Abacus, 2026; EPA, 2024a). |
| Bedrock Geology |  |

## Receptor Distances

<INSERT_25_TABLE>

| Surface Water | 270 m north |
| --- | --- |
| Water Well (Domestic/Stock) | 1012 m north (Well ID: 232262) |
| Dugout | 1800 northeast |
| Building or Residential Area | 520 m west |

## Soil Characteristics

<INSERT_26_TABLE>

| Texture | Fine / Coarse |
| --- | --- |
| Stratigraphy | Brief Description of soils based on bh logs and x-section |

## Groundwater Characteristics

<INSERT_27_TABLE>

| Water Table | 7.5 mbgs |
| --- | --- |
| Domestic Use Aquifer | 7.5 mbgs |
| Comments | No evidence of groundwater was observed to the maximum depth of investigation. Groundwater flow direction is unknown. |

## Exposure Pathway Evaluation

<INSERT_28_INTRO_START>

The following shows the relevant exposure pathways and receptors for agricultural land use, using Tier 1.

<INSERT_28_INTRO_END>

<INSERT_28_AG_STANDARD_TABLE>

Agricultural – Standard CoPCs – feel free to keep this, even if the site only has salinity data

| Exposure Pathway | CoPC | Comment |
| --- | --- | --- |
| Human |  |  |
| Direct soil contact | PHC, metals | Operable |
| Vapour inhalation (basement & slab) | PHC | Operable |
| Domestic use aquifer | PHC, boron, chloride* | Operable |
| Ecological |  |  |
| Direct soil contact | EC, SAR, pH, PHC, metals, chloride* | Operable |
| Nutrient/Energy cycling | Metals | Operable |
| Livestock soil & food ingestion | PHC, metals | Operable |
| Wildlife soil & food ingestion | PHC, barium | Operable |
| Freshwater aquatic life | PHC, boron, chloride* | Operable |
| Livestock water | PHC, boron, chloride* | Operable |
| Wildlife water | PHC, boron | Operable |
| Irrigation water | Boron, chloride* | Operable |
| Other |  |  |
| Management limit | PHC, chloride* | Operable |

*Although there is no specific guideline for soil chloride, it is a potential CoPC for the above pathways.

<INSERT_28_AG_ALL_TABLE>

Agricultural – ALL CoPCs

| Exposure Pathway | CoPC | Comment |
| --- | --- | --- |
| Human |  |  |
| Direct soil contact | PHC, PAH, other organics, metals | Operable |
| Vapour inhalation (basement & slab) | PHC, PAH, other organics | Operable |
| Domestic use aquifer | PHC, PAH, other organics, boron, chloride* | Operable |
| Ecological |  |  |
| Direct soil contact | EC, SAR, pH, PHC, PAH, other organics, metals, chloride* | Operable |
| Nutrient/Energy cycling | Metals, ethylene glycol | Operable |
| Livestock soil & food ingestion | PHC, PAH, metals | Operable |
| Wildlife soil & food ingestion | PHC, PAH, barium | Operable |
| Freshwater aquatic life | PHC, PAH, other organics, boron, chloride* | Operable |
| Livestock water | PHC, boron, chloride* | Operable |
| Wildlife water | PHC, boron | Operable |
| Irrigation water | Boron, chloride* | Operable |
| Other |  |  |
| Management limit | PHC, methanol, chloride* | Operable |

*Although there is no specific guideline for soil chloride, it is a potential CoPC for the above pathways.

<INSERT_28_NATURAL_STD_TABLE>

Natural Area land use – Standard CoPCs – feel free to keep this as a default for any site that doesn’t have PAHs, other organics, etc.

| Exposure Pathway | CoPC | Comment |
| --- | --- | --- |
| Human |  |  |
| Domestic use aquifer | PHC, boron, chloride* | Operable |
| Ecological |  |  |
| Direct soil contact | EC, SAR, pH, PHC, metals, chloride* | Operable |
| Nutrient/Energy cycling | Metals | Operable |
| Livestock soil & food ingestion | PHC, metals | Operable |
| Wildlife soil & food ingestion | PHC, barium | Operable |
| Freshwater aquatic life | PHC, boron, chloride* | Operable |
| Wildlife water | PHC, boron | Operable |
| Other |  |  |
| Management limit | PHC, chloride* | Operable |

*Although there is no specific guideline for soil chloride, it is a potential CoPC for the above pathways.

<INSERT_28_NATURAL_ALL_TABLE>

Natural Area land use – All CoPCs

| Exposure Pathway | CoPC | Comment |
| --- | --- | --- |
| Human |  |  |
| Domestic use aquifer | PHC, PAH, other organics, boron, chloride* | Operable |
| Ecological |  |  |
| Direct soil contact | EC, SAR, pH, PHC, PAH, other organics, metals, chloride* | Operable |
| Nutrient/Energy cycling | Metals | Operable |
| Livestock soil & food ingestion | PHC, PAH, metals | Operable |
| Wildlife soil & food ingestion | PHC, PAH, barium | Operable |
| Freshwater aquatic life | PHC, PAH, other organics, boron, chloride* | Operable |
| Wildlife water | PHC, boron | Operable |
| Other |  |  |
| Management limit | PHC, methanol, chloride* | Operable |

*Although there is no specific guideline for soil chloride, it is a potential CoPC for the above pathways.

# Guideline Selection

Soil analyses were compared with the following regulatory guidelines.

Alberta Tier 1 Soil and Groundwater Remediation Guidelines (Tier 1) (EPA, 2024a)

Salt Contamination Assessment and Remediation Guidelines (SCARG) (AENV, 2001)

Assessing Drilling Waste Disposal Areas: Compliance Options for Reclamation Certification (ADWDA) (AER, 2014)

Tier 1 with Background

## Tier 1

The following Tier 1 guideline categories were applied:

Land Use: Agricultural

Soil Type: Fine-grained / Coarse-grained or Fine and coarse-grained

Soil Depth: Surface soil

## SCARG

<INSERT_3_2_INTRO_START>

As all background samples were within the Good soil category for EC and SAR, these guidelines were applied to all characterization samples and depths as per Tier 1 (EPA, 2024a).

OR

A background statistical evaluation was completed to derive site-specific EC and SAR guidelines. The background statistics are summarized below and included in Appendix E.

<INSERT_3_2_INTRO_END>

Table 1: SCARG Guidelines

<INSERT_SCARG_TABLE>

| Depth | Guideline Category | Soil Rating Category |  | Guideline Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  | EC | SAR | EC | SAR |
| m |  | -- | -- | dS/m | no units |
| 0–0.3 | Topsoil | Good | Good | 2 | 4 |
| 0.3–1.0 | Subsoil | Good | Good | 3 | 4 |
| 1.0–1.5 | Subsoil | Good | Good | 3 | 4 |
| 1.5–3.0 | Subsoil | Poor | Fair | 10 | 8 |
| 3.0–4.5 | Subsoil | Poor | Fair | 10 | 8 |

<INSERT_33_START>

## Tier 1 with Background

Based on the background conditions, the following guidelines were applied.

Table 2: Tier 1 with Background Guidelines

<INSERT_33_TABLE>

| Media | Parameter | Tier 1 | Tier 1 with Background | Reference |
| --- | --- | --- | --- | --- |
| Soil | pH | 6–8.5 | 5.2–8.5 | Section 4.2.1 |

<INSERT_33_END>

# Soil Assessment Results

<INSERT_4_INTRO_START>

A Phase 2 ESA was conducted on November 2, 2026, advancing XX boreholes to depths of 1.5- 9.0 mbgs using a truck-mounted drilling rig with a 6-inch solid-stem auger. Borehole placement targeted areas of potential environmental concern identified in the historical investigation data.

The borehole locations and exceedance evaluation are shown in Figure XX; and the borehole logs, laboratory reports, safety protocols and sampling methodology are provided in Appendices A-C. The soil analytical results are summarized in Tables XX.

<INSERT_4_INTRO_END>

## Borehole Rationale

A summary of the borehole rationale is provided below:

Table 3: Borehole Rationale

<INSERT_41_TABLE>

| Borehole | APEC | Salinity | Metals | PHC | PAH | Other |
| --- | --- | --- | --- | --- | --- | --- |
| BG26-01, BG26-02 | Background | ✔ | - | - | - | - |
| BH26-01, BH26-02 | Well centre | ✔ | ✔ | ✔ | - | - |
| BH26-03 to BH26-08, BH26-10, BH26-11- | DWDA | ✔ | ✔ | ✔ |  | - |
| BH26-14 to BH26-18 | Flare pit | ✔ | ✔ | ✔ | ✔ | Glycol, methanol- |
| BH26-09, BH26-12, BH26-13 | EM Anomaly | ✔ | - | - | - | - |

## Site Characterization

### Background

<INSERT_421_START>

Two boreholes (BG26-01 and BH26-02) were advanced in undisturbed areas away from former facility locations and known contaminant sources to characterize background conditions and delineate site impacts. Eight samples were analyzed for salinity.

OR

Four background boreholes (BH26-19, BH26-22 to BH26-24) were advanced in undisturbed locations away from former facility locations and known contaminant sources to characterize background conditions and delineate site impacts. At three locations (BH26-19, BH26-23, and BH26-24), chloride concentrations exceeding the Tier 1 screening value of 100 mg/L were identified, and an evaluation and justification of the elevated background chloride is provided in Section 5.

AS REQUIRED

Subsequently, BH26-19 was excluded as a background location while the remaining boreholes were retained.

For this assessment, the background evaluation and guidelines are considered preliminary until further investigation is conducted.

Summary of Results

Both EC and SAR were in the Good soil rating category.

OR

EC values ranged from the Good to Fair soil rating categories.

SAR values were within the Good SCARG soil rating category.

Chloride concentrations ranged from 4 to 200 mg/L (3 to 118mg/kg).

FOR T1 with BG

- Naturally elevated soil pH, to a maximum of 8.7, was identified in BH25-03 at multiple depth increments. As such, the maximum background was applied as the Tier 1 with Background guideline (Section 3.3).

EC and SAR guidelines were applied at depth-based intervals, as outlined in Section 3.2 and Appendix E, to account for soil salinity concentrations that vary with depth.

OR

As all background samples were within the Good soil category for both EC and SAR, the default guidelines were applied to all characterization samples and depths.

<INSERT_421_END>

<INSERT_42_APECS_START>

### Well Centre - Clean

Two boreholes (BH26-01 and BH26-02) were advanced near the former wellhead to a maximum depth of X mbgs. Two samples were analyzed for some or all of the following:

Salinity

PHCs

Tier 1 regulated metals

Summary of Results

Both EC and SAR were in the Good soil rating category, with no SCARG exceedances.

OR

EC values ranged from the Good to Fair soil rating categories, with no SCARG exceedances.

SAR values were within the Good soil rating category, with no SCARG exceedances.

Chloride concentrations ranged from 4.9 to 5.7 mg/L (XX to XX mg/kg).

All other parameters complied with Tier 1.

OR

Chloride concentrations ranged from 4.9 to 300 mg/L (XX to XX mg/kg) and exceeded the 100 mg/L delineation guideline at boreholes BHxx-yy (xx to xx mbgs)

All other parameters complied with Tier 1.

Vertical closure to 100 mg/L chloride or background chloride concentration of XX mg/kg was achieved at the worst-case borehole (BH23-02) at 5.0 mbgs.

OR

Vertical closure to 100 mg/L for chloride was achieved at BH23-01 and BH23-03 at 2.0 and 3.0 mbgs, respectively.

OR

Vertical closure to 100 mg/L for chloride was achieved at BH23-01 at 2.0 mbgs but not at the two worst-case locations. At BH23-03 and BH23-04, residual chloride concentrations of 210 mg/L and 400 mg/L were identified at 5.0 mbgs.

### Well Centre - Impacted

Two boreholes (BH26-01 and BH26-02) were advanced near the former wellhead, to a maximum depth of x mbgs. Two samples were analyzed for some or all of the following:

Salinity

PHCs

Tier 1 regulated metals

Summary of Results

The soil pH value in one sample (reference BH and depth) exceeded the guideline range of 6.0–8.5, with a value of 8.64.

Both EC and SAR were in the Good soil rating category, with no SCARG exceedances.

OR

EC values ranged from the Fair to Unsuitable soil rating categories, with eight SCARG exceedances (reference depth of exceedances).

SAR values ranged from the Fair to Unsuitable soil rating categories, with eight SCARG exceedances (reference depth of exceedances).

Chloride concentrations ranged from 67 to 760 mg/L (XX to XX mg/kg), with six sample concentrations greater than 100 mg/L (reference depths of impact and boreholes).

All other parameters complied with Tier 1.

Vertical and lateral delineation for chloride was not achieved, although vertical delineation of the SCARG exceedances was achieved at 2.0 m.

### Well Centre – Justified EC

Two boreholes (BH20-1 and BH20-2) were advanced near the former wellhead, to a maximum depth of 3.0 mbgs. Two samples were analyzed for some or all of the following:

Salinity

PHCs

Tier 1 regulated metals

Summary of Results

Both EC and SAR values were in the Good soil rating category, with no SCARG exceedances.

OR

EC values ranged from the Good to Poor soil rating categories, with one SCARG exceedance.

SAR values were in the Good soil rating category, with no SCARG exceedances.

Chloride concentrations ranged from <5 to 11 mg/L (XX to XX mg/kg).

Vertical closure to 100 mg/L for chloride was achieved at the worst-case borehole (BH23-02) at 5.0 mbgs.

For the EC exceedance at BH21-01 (2.0 m):.

  - The chloride concentration of 11 mg/L is comparable to background conditions.

  - The EC value is associated with elevated levels of naturally occurring sulphate.

  - The sample location is below the shallow root zone at 2.0 mbgs.

  - Based on the above, the elevated EC is due to naturally occurring conditions and is assumed to have little or no environmental risk.

  - All other parameters complied with Tier 1.

OR

EC values ranged from the Good to Poor soil rating categories, with one SCARG exceedance.

SAR values were within the Good soil rating category.

Chloride concentrations ranged from 25 to 120 mg/L (XX to XX mg/kg).

For the SAR exceedance at BH20-02, BH20-03 and BH20-04..

  - The chloride concentration of 100 mg/L (xx mg/kg) is comparable to background conditions in the Medicine Hat region, where naturally occurring chloride is present (MEMS, 2016). On-site background chloride concentrations ranged up to 110 mg/L (XX mg/kg).

  - The EC values are primarily associated with elevated levels of naturally occurring sulphate (2100–3500 mg/L).

  - The sample locations are below the shallow root zone at 2.0 mbgs.

  - Based on the above, the elevated EC values are due to naturally occurring conditions and are assumed to have little or no environmental risk.

All other parameters complied with Tier 1.

### Well Centre – Justified SAR

Two boreholes (BH26-1 and BH26-2) were advanced near the former wellhead, to a maximum depth of 3.0 mbgs. Two samples were analyzed for some or all of the following:

Salinity

PHCs

Tier 1 regulated metals

Summary of Results

Both EC and SAR were in the Good soil rating category, with no SCARG exceedances.

OR

EC values were in the Good soil rating category, with no SCARG exceedances.

SAR values ranged from the Good to Fair soil rating categories, with one SCARG exceedance.

Chloride concentrations ranged from <5.2 to 14 mg/L (XX to XX mg/kg).

For the SAR exceedance at BH25-07 (0.6 m):

  - The chloride concentration of 42 mg/L is comparable to background conditions.

  - The elevated SAR is attributed to an ion imbalance caused by naturally occurring sodium sulphate, in combination with comparably low calcium and magnesium concentrations.

  - The sample location is below the shallow root zone at 3.0 mbgs.

  - Based on the above, the elevated SAR is due to naturally occurring conditions and is assumed to have little or no environmental risk.

All other parameters complied with Tier 1.

### Separator/Methanol Tank

One borehole (BH26-03) was advanced near the former separator and methanol tank, to a depth of 3.0 mbgs. Two characterization samples were analyzed for some or all of the following:

Salinity

PHCs

Methanol

Summary of Results

Both EC and SAR values were in the Good soil rating category, with no SCARG exceedances.

The chloride concentration was 11 mg/L (8 mg/kg).

Methanol exceeded in both samples, at depths of 1.25 and 3.0 mbgs. Delineation was not achieved.

All other parameters complied with Tier 1.

### Infrastructure #2

One borehole (BH26-04) was advanced within the Infrastructure #2 area to a depth of 1.5 mbgs. The characterization sample was analyzed for the following:

Salinity

PHCs

Summary of Results

Both EC and SAR values were in the Good rating category, with no SCARG exceedances.

The chloride concentration was <3 mg/L (<2 mg/kg).

One sample, BH25-08 (0.6 m) exceeded for PHC F2 to F4. Vertical delineation was achieved at 1.25 mbgs, and lateral delineation was sufficient.

All other parameters complied with Tier 1.

### Infrastructure #3

One borehole (BH26-05) was advanced within the Infrastructure #3 area to a depth of 1.5 mbgs. The characterization sample was analyzed for the following:

Salinity

PHCs

Summary of Results

Both EC and SAR values were in the Good soil rating category, with no SCARG exceedances.

The chloride concentration was 5 mg/L (2 mg/kg).

All other parameters complied with Tier 1.

### Surface Staining

One borehole (BH26-06) was advanced within the surface staining area observed during the Phase 1 ESA (TLS, 2023) to a depth of 1.5 mbgs. The characterization sample was analyzed for the following:

Salinity

PHCs

Summary of Results

Both EC and SAR values were in the Good soil rating category, with no SCARG exceedances.

The chloride concentration was 5 mg/L (2 mg/kg).

All other parameters complied with Tier 1.

- <INSERT_42_APECS_END>

- <INSERT_42_DWDA_START>

### Drilling Waste Disposal Area

As the wellbore depth was less than 1500 m, three representative samples of the DWDA soil-waste mix zone (SWMZ) are required (AER, 2014). Four exploratory boreholes (BH26-01 to BH26-04) were advanced to a maximum depth of 3.0 mbgs at the most likely locations for a DWDA, based on the site layout, landscape position, topography, and historical drilling practices. The SWMZ was identified and characterized in three boreholes (BH26-05 to BH26-07) and further delineated in two boreholes (BH26-08 and BH26-09). Per the Phase 1 Compliance Options, each characterization sample was analyzed for the following:

- Salinity

- PHCs

- Tier 1 regulated metals

Summary of Results

The soil waste mix zone was identified in BH26-XX (XX to XX mbgs), …

Both EC and SAR were in the Good soil rating category, with no SCARG exceedances.

OR

EC values ranged from the Good to Poor soil rating categories, with no SCARG exceedances.

SAR values ranged from the Good to Fair soil rating categories, with no SCARG exceedances.

Chloride concentrations ranged from 6.8 to 80 mg/L (XX to XX mg/kg).

All other parameters complied with Tier 1.

- <INSERT_42_DWDA_END>

- <INSERT_42_DWDA_NIL_START>

### Drilling Waste Disposal Area – NIL FINDINGS

As the wellbore depth was less than 1500 m, three representative samples of the DWDA soil-waste mix zone are required (AER, 2014). Five boreholes (BH26-XX to BH26-XX) were advanced to a maximum depth of x mbgs in the most likely locations for a DWDA based on the site layout, landscape position, topography, and historical drilling practices. No indications of soil disturbance, drilling waste, or contaminant impacts were identified in any of the exploratory boreholes. As such, samples were not collected or analyzed for this APEC assessment. It was concluded that a DWDA is not present or is of such low volume as to pose little to no environmental risk. The DWDA search locations are shown in Figure 2.

OR – with an EM survey

As the wellbore depth was less than 1500 m, three representative samples of the DWDA soil-waste mix zone are required (AER, 2014). Five boreholes (BH26-XX to BH26-XX) were advanced to a maximum depth of x mbgs at the most likely locations for a DWDA, based on the site layout, landscape position, topography, and historical drilling practices. Additional consideration was given to the geophysical surveys (AKS, 2020), which did not show a significant response indicative of a drilling waste disposal area (Appendix C).

No indications of soil disturbance, drilling waste, or contaminant impacts were identified in the exploratory boreholes. As such, samples were not collected or analyzed for this APEC assessment. It was concluded that a DWDA is either absent or of such low volume that it poses little to no environmental risk. The DWDA search locations are shown in Figure 2.

OR – with an EM survey that shows an anomaly outside the typical region for a sump.

As the wellbore depth was less than 1500 m, three representative samples of the DWDA soil-waste mix zone are required (AER, 2014). Five boreholes (BH26-XX to BH26-XX) were advanced to a maximum depth of x mbgs at the most likely locations for a DWDA based on the site layout, landscape position, topography, and historical drilling practices. Additional consideration was given to the geophysical surveys (AKS, 2020), which did not indicate elevated conductivity representative of a drilling waste disposal area (Appendix C). In the Medicine Hat region, drilling waste disposal areas are characterized by shallow impacts near the wellbore.

No indications of soil disturbance, drilling waste, or contaminant impacts were identified in any of the exploratory boreholes. As such, samples were not collected or analyzed for this APEC assessment. It was concluded that a DWDA is not present or is of such low volume as to pose little to no environmental risk. The DWDA search locations are shown in Figure 2.

- <INSERT_42_DWDA_NIL_END>

## Tier 1 Chloride Delineation

<INSERT_43_START>

Example with background chloride:

Lateral and vertical delineation to the Tier 1 standard of 100 mg/L chloride was achieved at the well centre area (Figure 5). For the site as a whole, elevated background chloride was identified at BH25-24, south of the site, and at BH25-25, east of the site, as discussed further in Section 5. Vertical delineation was achieved at each location with elevated chloride (BH26-02, BH26-24, and BH26-25), as shown in Figure 1.

Example with background chloride

Lateral delineation to the Tier 1 standard of 100 mg/L chloride was not achieved due to the naturally occurring chloride identified at BH26-06 (Section 6). However, vertical delineation was achieved at BH26-01, and field screening results from the surrounding boreholes do not indicate any chloride impacts. Additionally, the EM31 survey showed no indication of on-site hotspots.

Example with significant chloride impacts

Lateral chloride delineation to the Tier 1 standard of 100 mg/L chloride was achieved to the west and south, although not at equivalent depths to the worst-case impacts. Lateral delineation was not achieved to the east and north (Figure 5), and vertical delineation was not achieved at the EM anomaly/former flare pit, the separator and UST, and the south flare pit.

The worst-case chloride concentrations were observed in the southern portion of the site, at the EM anomaly and suspected flare pit (BH25-01), where concentrations as high as 3830 mg/L were measured at 7.5 mbgs.

For cases where delineation was not achieved due to too little assessment or analytical information.

No indications of soil disturbance, drilling waste, or contaminant impacts were identified in any of the exploratory boreholes, and all samples had chloride concentrations below the Tier 1 chloride delineation screening value of 100 mg/L. The maximum chloride concentration at each borehole location is illustrated in Figure 2.

OR - NIL findings report

No indications of soil disturbance, drilling waste, or contaminant impacts were identified in any of the exploratory boreholes.  As such, samples were not collected or analyzed.

OR

Lateral delineation to the Tier 1 standard of 100 mg/L chloride was achieved in all cardinal directions, as illustrated by the 100 mg/L contour in Figure 2. Vertical delineation of chloride impacts is shown in Table 4 and in Figure 1 below, which presents the vertical soil chloride profile.

OR

Lateral delineation was achieved relative to local background conditions; however, concentrations exceeded the Tier 1 standard of 100 mg/L. A comprehensive analysis of the on-site background chloride is provided in Section 5, referencing the naturally occurring chloride, which is prevalent in the Medicine Hat region.

For this site, the delineation criteria of 157 mg/kg was used to define the impact area, illustrated by the 157 mg/kg contour in Figure 2. The vertical delineation of chloride impacts is shown in Table 4 and in Figure 1 below, which presents the vertical soil chloride profile.

<INSERT_43_END>

Figure 1. Soil Chloride Profile

<INSERT_43_NA_PROFILE_START>

<INSERT_43_NA_PROFILE_END>

# Background Chloride

<INSERT_5_INTRO_START>

As noted in Section 4.2.1, boreholes were advanced in undisturbed areas, away from former facility locations and known contaminant sources, to characterize background soil chemistry and native soil profiles.

Boreholes BH26-01, BH26-10, BH26-15 to BH26-18 and BH26-22 were advanced near the site boundaries, with no evidence of soil disturbance noted in the borehole logs. However, chloride concentrations exceeding the Tier 1 delineation guideline of 100 mg/L were identified at BH26-01, BH26-15, BH26-16, and BH26-18, with two maximum values of 150 mg/kg at BH26-15 (2.0 m) and BH26-16 (3.0 m).

As naturally occurring chloride associated with saline lakes and seeps is well documented in the region (Last & Ginn, 2005; Kwiatkowski, et al., 1996; Vance et al., 1993), a review of the historical activities and site-specific soil and groundwater information was completed to evaluate the elevated chloride in the outlying background locations.

<INSERT_5_INTRO_END>

## Historical Site Activities

<INSERT_5_1_START>

The Phase 1 ESA was reviewed for potential sources of chloride impact in the outlying areas, and the following was noted (Shark, 2007):

There are no other wellsites, facilities, municipal roads, or highways near the site.

There were no recorded spills associated with the site or nearby pipelines.

A drilling flare pit was likely in place but not used, as drilling was completed within two days, with no kicks or upsets.

During production, the well produced a nominal volume (4.9 m3) of produced water over four years before ceasing operation in 1986.

There are two pipelines associated with the site, which head east and southeast from the well centre.

The area is likely used for cattle grazing, which could result in some salt-related impacts at the near-surface.

The background borehole locations are shown in Figure XX, overlaid onto a post-disturbance aerial photograph from 1980. Each background location is at least 14 m beyond the area of historical disturbance, with no other known sources in proximity. Two potential exceptions would be BH26-15 and BH26-16, which are in the general vicinity of the two pipelines (Figure XX).

<INSERT_5_1_END>

## Contaminant Transport Mechanisms

<INSERT_5_2_START>

Potential contaminant transport mechanisms were evaluated in relation to the elevated chloride in the outlying boreholes.

The sulphate profile evaluation (Section X) suggests that the vertical gradient is downwards toward the shallow water table or DUA, which is consistent with the arid climate of the Dry Mixedgrass natural subregion.

The borehole logs (Appendix B) show predominantly fine-grained soils, which is consistent with the loam to clay loam texture analysis presented in Table XX.

The borehole logs indicate that significant mottling, gleying, or saturated intervals were not observed to the maximum investigation depth of 9.0 mbgs, the current estimate of the shallow groundwater table.

At the impacted well centre and DWDA, elevated chloride concentrations are concentrated in the root zone (maximum of 2800 mg/kg), extending from approximately 0.4 to 6.0 mbgs.

At the background locations, elevated chloride concentrations (100 to 150 mg/kg) were identified from 2.0 to 4.5 mbgs, to the northwest (BH26-01 and BH26-18), east (BH26-15), and southeast (BH26-16) of the known impact area on-site.

It is expected that chloride impacts from the well centre and DWDA will leach downward in the unsaturated zone towards the groundwater table. Impacts that reach the groundwater table would move laterally downgradient with the groundwater flow.

Based on the depth of soil impact on-site relative to the groundwater table, the probability of groundwater impacts is relatively low. Consequently, lateral transport from the source area to the outlying areas is extremely low. Despite this, neither of these transport mechanisms would account for chloride impacts at the depths (2.0–4.5 m) and locations encountered in the background boreholes, above the groundwater table and in multiple directions from the source area.

<INSERT_5_2_END>

## Chemistry Footprint

<INSERT_5_3_START>

A simple comparison between the source and background areas was conducted by evaluating the ratios of chloride to sulphate and potassium. Sulphate is a naturally occurring salt that is prevalent in the region, and potassium is a common drilling mud additive, as identified in the Phase 1 ESA (Shark, 2007). For a more representative comparison, the units for each comparison are in meq/L.

Figure 1 shows the relationship between potassium and chloride. As expected, samples from the impact area contain elevated levels of potassium and chloride. In contrast, the background samples contain equally low potassium levels regardless of the elevated chloride.

Figure 2 shows a striking contrast between the impact area and the background samples. The background locations with elevated chloride (yellow) are grouped vertically with the background samples, whereas those from the impact area (red) are grouped separately.

<INSERT_5_3_FIGURES_START>

Figure 2: Chemistry Footprint: Potassium vs. Chloride

Figure 3: Chemistry Footprint: Sulphate vs. Chloride

<INSERT_5_3_FIGURES_END>

<INSERT_5_3_END>

## Background Chloride Summary

<INSERT_5_4_START>

Based on the above lines of evidence, it is concluded that the elevated chloride levels at the outlying background locations are not related to the on-site source area and are due to naturally occurring conditions. As such, boreholes BH25-08 to BH25-11 were used as background for SCARG guideline development.

<INSERT_5_4_END>

# Impact Volume

Below is a summary of the preliminary remediation volume, shown in Figure XX. The estimates do not account for justified exceedances.

Volume-to-tonnage conversions were based on a soil bulk density of 1.7 t/m3, and a 10% contingency was applied.

For a rough estimate with little data

Volume-to-tonnage conversions were based on a soil bulk density of 1.7 t/m3. Due to the absence of a detailed delineation, the lateral and vertical dimensions are approximate, and a 15% contingency was applied.

Lack of delineation:

Volume-to-tonnage conversions were based on a soil bulk density of 1.7 t/m3. As vertical and lateral delineation is incomplete for the compressor area, a 20% contingency was applied.

Table 4: Remediation Volume Estimate

| Area | Area | Top | Base | Volume | Tonnes |
| --- | --- | --- | --- | --- | --- |
|  | m | mbgs | mbgs | m3 | t |
| Well centre | 100 | 0 | 3 | 300 | 540 |
| DWDA | 200 | 1 | 3 | 400 | 720 |
| Subtotal |  |  |  | 700 | 1260 |
| Contingency (15%) |  |  |  | 105 | 189 |
| Total |  |  |  | 805 | 1449 |

Table with Impact Type added.

| Area | Impact Type | Area | Top | Base | Volume | Tonnes |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | m | mbgs | mbgs | m3 | t |  |
| Well Centre | Salinity | 100 | 0 | 3 | 300 | 540 |  |
| DWDA | PHC and metals | 200 | 1 | 3 | 400 | 720 |  |
| Subtotal |  |  |  |  | 700 | 1260 |
| Contingency (15%) |  |  |  |  | 105 | 189 |
| Total |  |  |  |  | 805 | 1449 |

Accounting for re-used soil

Currently, there are opportunities to reuse marginally impacted soil from portions of the excavation for backfill in another.

Select soils from the DWDA (0–1.0 m) may be reused as backfill at the bottom of the excavation, from 3.0–4.0 m.

Select soils from the well centre area may be reused as potential backfill at the bottom of the DWDA excavation from 3.0 to 4.0 m.

Large areas of potential impact have not been well characterized, and additional opportunities for backfill replacement exist.

| Area | Area | Top | Base | Volume | Tonnes |
| --- | --- | --- | --- | --- | --- |
|  | m | mbgs | mbgs | m3 | t |
| Well centre | 100 | 0 | 3 | 300 | 540 |
| DWDA | 200 | 1 | 3 | 400 | 720 |
| Excavation Subtotal |  |  |  | 700 | 1260 |
| Re-used soil from the DWDA |  |  |  | 100 | 180 |
| Landfill Total |  |  |  | 600 | 1080 |

# Compliance Summary

## APEC Review

The following APECs were investigated for the corresponding CoPCs.

Table 5: APEC and COPC Investigation Summary

<INSERT_71_TABLE>

| APEC | Salinity | Metals | PHC | PAH | Other | Meets Applicable Guidelines and/or Justified |
| --- | --- | --- | --- | --- | --- | --- |
| Well centre | ✔ | ✔ | ✔ | - | - | Yes |
| DWDA | ✔ | ✔ | ✔ | - | - | No |
| Flare pit | ✔ | ✔ | ✔ | ✔ | - | Yes |
| Flare Stack | ✔ | - | ✔ | ✔ | - | Yes |
| Pit | ✔ | ✔ | ✔ | - | - | Yes |
| EM Anomaly | ✔ | - | - | - | - | Yes |

## Justification Summary

<INSERT_72_JUSTIFICATION_START>

Justifications were not required for this assessment.

OR

Below is the complete list of the justifications for this assessment.

Table 6: Concordance Table

| Issue | Justification | Rationale | Reference |
| --- | --- | --- | --- |
| Background SAR | Procedural | One background SAR value exceeded the 95th percentile applied to the 1–1.5 m interval. This sample is not associated with chloride impact, and the exceedance results solely from the SCARG calculation method, which does not apply the maximum value. | Section XX |
| EC and SAR | Minor exceedance | Minor exceedance justifications were made for XX EC and XX SAR exceedances with moderate chloride concentrations less than 300 mg/L (or 300 mg/kg). The exceedances were primarily driven by sulphate concentrations ranging from 2180-3540 mg/L. For each respective borehole, the salinity exceedances and chloride concentrations were vertically delineated. As such, the exceedances are primarily driven by natural conditions and pose a low environmental risk. | Section XX |
| EC and SAR | Minor exceedance | Minor exceedance justifications were made for XX EC and XX SAR exceedances with chloride concentrations less than 100 mg/L. Because of naturally occurring sodium sulphate on-site, the exceedances are primarily driven by natural conditions and pose a low environmental risk. | Section XX |
| Chloride in Background | Procedural | Elevated chloride concentrations were identified in the outlying background boreholes. A review of the historical activities and site-specific information concluded that the concentrations were due to naturally occurring conditions. | Section XX |

<INSERT_72_JUSTIFICATION_END>

Table 7: Residual Site Exceedances

<INSERT_72_RESIDUAL_TABLE>

| Location | Depth (m) | APEC | Parameter | Value | Guideline | Chloride (mg/L) | Section | Figure | Table |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BH119-11 | 0.75 | DWDA | EC (dS/m) | 3.6 | 3.0 | 4 | 4.22 | 2 | 4 |
| 14BH02 | 0.15-0.6 | Well Centre | SAR | 4.2 | 4.0 | 17 | 4.22 | 2 | 4 |
| 14BH07 | 0.6-1.0 | EM anomaly | pH | 4.1 | 5.8 | 13 | 7.1 | 2 | 4 |
| BH112-11 | 3.75 | Well Centre | Se (mg/kg) | 1.3 | 1.0 | 13 | 7.2 | 4 | 6 |
| BH130-11 | 5.25 | South AST | F2 (mg/kg) | 2.9 | 1.0 | 13 | 7.2 | 3 | 5 |

Note: Non-justified exceedances are noted in red text.

# Summary and Conclusions

<INSERT_8_START>

The following summarizes the results of the Phase 2 ESA at the 07-06-020-27 W4M wellsite.

Background SCARG

Background samples were not collected, and the Good soil rating category was applied to both EC and SAR.

- OR

The background evaluation identified the appropriate soil rating categories as Good for both EC and SAR.

- OR

The background evaluation identified the appropriate soil rating categories as Poor for EC and Fair for SAR.

- OR

The background evaluation identified the appropriate soil rating categories as Good to Poor for EC and Fair to Unsuitable for SAR.

- Background Chloride Summary

Marginally elevated chloride concentrations were identified in background. A site-specific evaluation and justification are provided in Section X.

- Tier 1 Exceedances Summary

No soil exceedances were identified in the well centre and DWDA assessments.

- OR

Soil exceedances were identified in the well centre and DWDA assessments. However, all of the exceedances were assumed to be naturally occurring due to the low chloride concentrations.

All other parameters complied with the applicable Tier 1 guidelines.

- OR

No soil exceedances were identified in the well centre assessment.

Salinity and hydrocarbon exceedances were identified in the DWDA and EM anomaly assessments.

Minor exceedance justifications were applied to select samples in the DWDA due to low chloride concentrations.

All other parameters complied with the applicable Tier 1 guidelines.

- OR

Soil exceedances were identified in the well centre and DWDA assessments. However, all of the exceedances were assumed to be naturally occurring due to the low chloride concentrations.

All other parameters complied with the applicable Tier 1 guidelines.

- OR

No evidence of impact or soil disturbance was identified in any of the exploratory boreholes. Therefore, samples were not collected or analyzed as part of the DWDA investigation.

All other parameters complied with the applicable Tier 1 guidelines.

- Chloride Delineation - examples with increasing complexity

Chloride concentrations for all of the characterization samples were less than 100 mg/L and delineation was not required.

Chloride delineation to the Tier 1 standard of 100 mg/L was achieved.

Chloride delineation to the Tier 1 standard of 100 mg/L, or background levels, was achieved.

Vertical delineation to the Tier 1 standard of 100 mg/L was achieved, however, lateral delineation was not achieved to the south and west of the site.

Vertical delineation to the Tier 1 standard of 100 mg/L was achieved at the well centre and DWDA, but not at the flare pit and compressor areas. Lateral delineation was achieved in all directions except south, which can be inferred from the geophysical survey.

- Compliance statement

All known APECs have been adequately characterized for the appropriate CoPCs. A comprehensive compliance review is provided in Section 7.

Concluding statements

No exceedances, no delineation issues

The ESA has provided sufficient information to support that there is no reasonable basis to suspect a substance release has occurred at the site that has caused, is causing, or may cause an adverse effect. Therefore, no further remedial actions are required.

For sites with minor exceedance justifications

Minor exceedances were identified and have been justified based on environmental risk. As such, the ESA has provided sufficient information to support that there is no reasonable basis to suspect a substance release has occurred at the site that has caused, is causing, or may cause an adverse effect. Therefore, no further remedial actions are required.

For impacts requiring remediation

The ESA has confirmed that a substance release has occurred at the site, and further assessment, remedial measures, or exposure control measures are required. The impact volume is currently estimated at 696 m3 (1253 tonnes).

When further work is required – unknown volume

The ESA has confirmed that a substance release has occurred at the site, and further assessment, remedial measures, or exposure control measures are required.

<INSERT_8_END>

# Closure

We trust that this report meets your requirements. For questions or concerns regarding the information, please contact Robert Wiedemann at rwiedemann@ebmgeoscience.com.

Respectfully submitted,

EBM Geoscience Inc.
APEGA Permit to Practice: 14970

Report written by:					Report reviewed by:

Beth Crowe, BSc (Hons), PAg			Robert Wiedemann, MEnvSc, PAg, PGeo

Senior Environmental Consultant			Senior Environmental Geoscientist

AIA: 5803						AIA: 2751. APEGA: 105310

OR

We trust that this report meets your requirements. If you have questions or concerns about the information, please contact Steve Vogt at svogt@ebmgeoscience.com.

Respectfully submitted,

EBM Geoscience Inc.

Report written by:						Report reviewed by:

Jeff Brown, BSc (Hons), EP, AIT 				Steve Vogt, BSc, PAg

Environmental Consultant					Senior Project Manager

AIA: 24540							AIA: 4621

OR

We trust that this report meets your requirements. For questions or concerns regarding the information, please contact Jenifer Kelly at jkelly@embgeoscience.com.

Respectfully submitted,

EBM Geoscience Inc.
APEGA Permit to Practice: 14970

Report written by:					Report reviewed by:

Michelle Haager, MSc 				Jenifer Kelly, MSc, PEng

Risk Assessment Specialist				Senior Environmental Engineer

APEGA: 109506

# Limitations of the Report

This report was prepared by EBM Geoscience Inc. (EBM) for the exclusive use of Canlin Energy Corporation (the client) and may not be reproduced in whole or in part without the prior written consent of EBM or used or relied upon in whole or in part by a party other than mentioned in these Limitations. Any unauthorized use of this report, or any part thereof, by a third party, or any reliance on or decisions to be made based on it, is at the sole risk of such third parties. EBM accepts no responsibility for damages, if any, suffered by any third party as a result of decisions made or actions based on this report, in whole or in part.

Standard of Care and Disclaimer of Warranties. This study and report have been prepared in accordance with generally accepted environmental and scientific judgments, principles, and practices. EBM expressly disclaims any and all warranties in connection with this report, including, without limitation, any warranty that this report and the associated site investigation work have uncovered all potential environmental liabilities associated with the subject property.

Investigation and Subsurface Risks. The environmental characterization data were collected in general accordance with the standards and methods identified in the document by experienced professionals. Subsurface conditions between boreholes, monitoring wells, and sampling locations have been based, by necessity, on assumptions of what exists between the actual locations sampled or investigated and may vary significantly from actual site conditions. If applicable, interpretations of groundwater levels and flow direction are based on water level measurements at selected monitoring well locations and are expected to fluctuate. Borehole and monitoring well observations indicate the approximate subsurface conditions at those locations only. Even a comprehensive sampling and testing program, implemented in accordance with appropriate equipment by experienced personnel, may fail to detect certain conditions. Actual conditions may vary significantly between the points investigated, and all persons making use of this report should be aware of and accept this risk. Subsurface sampling may result in unavoidable contamination of certain subsurface areas not known to be previously contaminated, such as, but not limited to, a geologic formation, the groundwater or other hydrous bodies. EBM is not responsible for such contamination.

Information from Client and Third Parties. EBM has relied in good faith on information provided by the client and third parties noted in this report and has assumed such information to be accurate, complete, reliable, non-fringing, and fit for the intended purpose without independent verification. EBM accepts no responsibility for any deficiency, misstatements or inaccuracy contained in this report as a result of omissions or errors in the information provided by third parties or for omissions, misstatements or fraudulent acts of persons interviewed.

Independent Judgments. EBM will not be responsible for the independent conclusions, interpretations, interpolations, and/or decisions of the client or others who may come into possession of this report or any part thereof. This restriction of liability includes decisions made to purchase, finance, or sell land.

Effect of Changes. All evaluations and conclusions stated in this report are based on facts, observations, site-specific details, legislation, and regulations as they existed at the time of the investigation. Some conditions are subject to change over time, and the client recognizes that the passage of time, natural occurrences, and direct or indirect human intervention at or near the site may substantially alter such evaluations and conclusions. Regulatory statutes are also subject to change and interpretation, which may change over time. EBM should be requested to re-evaluate the conclusions of this report and to provide amendments as required prior to any reliance upon the information presented herein upon any of the following events: a) any changes (or possible changes) as to the site or regulatory requirements upon which this report was based, or b) new information is discovered in the future during site excavations, building demolition or other activities, or c) additional subsurface investigations or testing conducted by others.

# References

Abacus. (2026). Abacus Datagraphics Ltd. Retrieved from http://www.abacusdatagraphics.com/index.asp

AENV. (2001). Salt Contamination Assessment and Remediation Guidelines (Pub. No. T/606).

AEP. (2017). Field Preservation of Volatile Compounds with Methanol. Land Policy Branch, Policy and Planning Division: Alberta Environment and Parks.

AEP. (2018). Environmental Quality Guidelines for Alberta Surface Waters. Alberta Environment and Parks.

AER. (2014). Assessing Drilling Waste Disposal Areas: Compliance Options for Reclamation Certification. Alberta Energy Regulator.

AGS. (2013). Geological Map of Alberta. Scale 1:1,000,000. Edmonton, AB: Alberta Geological Survey and Alberta Energy Regulator.

AKS. (2020). EM38 and EM31 Conductivity Data 100/XX-XX-XXX-XX W4M Medicine Hat, Alberta. AKS Geoscience Inc.

EPA. (2024a). Alberta Tier 1 Soil and Groundwater Remediation Guidelines. Alberta Environment and Protected Areas.

EPA. (2024b). Environmental Site Assessment Standard. Alberta Environment and Protected Areas.

Kwiatkowski, et al. (1996). Salinity mapping for resource management.

Last, W. M., & Ginn, F. M. (2005). Saline systems of the Great Plains of western Canada; an overview of the limnogeology and paleolimnology. Saline Systems, 1, 1-38.

LEM. (2020). Phase 1 Environmental Site Assessment; Compton Gladys 7-6-20-27. Calgary, Alberta: Lawson Environmental Management Ltd.

MEMS. (2016). Regional Salt Risk Assessment in Southeastern Alberta Methodology Report. Millennium EMS Solutions Ltd.

Natural Regions Committee. (2006). Natural Regions and Subregions of Alberta. Compiled by D.J. Downing and W.W. Pettapiece: Government of Alberta. Pub. No. T/852.

Shark. (2007). Phase 1 ESA for 100/14-22-014-02 W4M. Shark Resource Technology Ltd.

TLS. (2023). Phase 1 Environmental Site Assessment For The Facility at 16-10-011-13 W4M License #F567 100/16-10-011-13W4M. TerraLogix Solutions Inc.

Vance et al. (1993). Holocene paleohydrology of a hypersaline lake in southeastern Alberta. Journal of Paleolimnology, 8, 103-120.

# Figures

# Tables

# Appendix A: Borehole Logs

# Appendix B: Laboratory Reports

# Appendix C: Geophysical Surveys

# Appendix D: Methodology

Safety

All personnel follow the appropriate safe work practices. Additional standards and requirements presented by the client and field personnel are reviewed, and the more stringent protocols are adopted.

EBM adheres to all ground disturbance policies and procedures before ground disturbance activities, according to standard operating procedures and the client’s requirements.

Soil Sampling

Soil samples are collected with an environmental drilling rig or a manual sampling device. Visual observations and the soil texture are recorded for each distinct soil interval. Sampling locations are recorded using a global positioning system (GPS) or physically measured in the field relative to a fixed location.

Where appropriate, field screening consisted of the following:

- Volatile organic compound (VOC) measurements with an RKI EagleTM portable gas monitor calibrated with hexane and methane elimination.

- EC by a Field Scout® Direct Soil EC Probe.

- Chlorides by soil solution filtration and Quantab titration strips.

Field screening and physical observations helped determine potential impacts, the vertical and lateral extent of the investigation, and the selection of samples for laboratory analysis.

Soil QA/QC

Before field use, the portable gas monitor and soil EC probe are calibrated according to the manufacturer's instructions. All field instruments and equipment are calibrated by the rental vendor or EBM as required.

Soil sampling is conducted in accordance with standard practices for assessing upstream oil and gas sites (EPA, 2024b). To minimize cross-contamination, soil samples are prepared by removing the outer smear when collected with an auger or hand tool. Only the inner portions of the sample are used for field screening and laboratory analysis. Soil sampling equipment that comes into contact with soil is scraped clean between sample locations, and nitrile gloves are changed for each sample.

Samples are preserved in laboratory-supplied containers and submitted to an accredited third-party laboratory for chemical analysis. Samples submitted for organic analyses are collected in jars with minimal headspace, and volatile parameters are field-preserved in methanol in accordance with current regulatory standards (AEP, 2017). Samples for inorganic parameters are placed in sealable plastic bags. Samples are labelled and transferred to laboratory custody using the standard Chain of Custody procedures within the appropriate holding time. During transport, samples are maintained at the appropriate temperature for analysis.

# Appendix E: Background Statistics

1	Background EC and SAR Statistics

<INSERT_APPE_START>

1.1	 Background Topsoil Data 0 to 0.3 m

| Borehole | Depth | pH | Sat | EC |  | SAR |  | Cl |  | SO4 |  | Na |  | Ca |  | Mg |  | K |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mbgs | --- | % | dS/m |  | --- |  | mg/kg |  |  |  |  |  |  |  |  |  |  |  |
| BH21-1 | 0.1 | 6 | 102 | 0.33 |  | 0.11 |  | 22.7 |  | 38.6 |  | 2.5 |  | 48.3 |  | 18 |  | 13 |  |
| BH21-10 | 0-0.12 | 5.9 | 170 | 0.68 |  | 0.36 |  | 54.8 |  | 291 |  | 20.6 |  | 79.3 |  | 38.2 |  | 111 |  |
| BH22-15 | 0.1 | 7.38 | 68 | 0.73 |  | 0.28 |  | 3.4 |  | 15 |  | 7.7 |  | 61 |  | 15 |  | 11 |  |
| BH22-16 | 0.1 | 6.86 | 68 | 0.39 |  | 0.31 |  | 3.4 |  | 20 |  | 6.4 |  | 31 |  | 10 |  | 13 |  |
| BH22-17 | 0.1 | 6.65 | 65 | 0.55 |  | 0.27 |  | 3.25 |  | 75 |  | 7.2 |  | 59 |  | 13 |  | 22 |  |
| BH22-18 | 0.1 | 7.31 | 62 | 0.75 |  | 0.24 |  | 3.1 |  | 32 |  | 6.1 |  | 60 |  | 13 |  | 12 |  |
| BH22-22 | 0.1 | 5.92 | 70 | 0.31 |  | 0.83 |  | 7.6 |  | 32 |  | 12 |  | 12 |  | 5.9 |  | 23 |  |
| Mean |  |  |  |  | 0.5 |  | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard Deviation |  |  |  |  | 0.2 |  | 0.2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 95th Percentile |  |  |  |  | 0.7 |  | 0.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| SCARG Category Rating |  |  |  |  | Good |  | Good |  |  |  |  |  |  |  |  |  |  |  |  |
| Guideline |  |  |  |  | 2 |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.2 	Background Shallow Rooting Zone Data 0.3 to 1.0 m

| Borehole | Depth | pH | Sat | EC |  | SAR |  | Cl |  | SO4 |  | Na |  | Ca |  | Mg |  | K |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mbgs | --- | % | dS/m |  | --- |  | mg/kg |  |  |  |  |  |  |  |  |  |  |  |
| BH21-1 | 0.4 | 7.5 | 68 | 0.38 |  | 0.12 |  | 2.5 |  | 15.8 |  | 2.5 |  | 36.3 |  | 13.5 |  | 2.5 |  |
| BH21-10 | 0.4 | 7.7 | 92 | 0.52 |  | 3.49 |  | 14.9 |  | 85.5 |  | 72 |  | 16.7 |  | 11.1 |  | 12.2 |  |
| BH22-15 | 0.4 | 7.76 | 51 | 0.31 |  | 0.65 |  | 2.55 |  | 6.5 |  | 9.6 |  | 18 |  | 8.7 |  | 2.7 |  |
| BH22-16 | 0.4 | 7.7 | 51 | 0.3 |  | 0.71 |  | 2.55 |  | 5.2 |  | 10 |  | 20 |  | 7.5 |  | 3 |  |
| BH22-17 | 0.4 | 7.74 | 47 | 0.33 |  | 0.78 |  | 2.35 |  | 8 |  | 10 |  | 15 |  | 8.1 |  | 2.9 |  |
| BH22-18 | 0.4 | 7.75 | 50 | 0.27 |  | 0.74 |  | 2.5 |  | 4.3 |  | 9.6 |  | 13 |  | 7.4 |  | 3.7 |  |
| BH22-22 | 0.4 | 7.65 | 68 | 0.91 |  | 5.6 |  | 20 |  | 170 |  | 91 |  | 13 |  | 9.7 |  | 9.3 |  |
| BH21-1 | 0.8 | 7.6 | 73 | 0.44 |  | 1.18 |  | 9.1 |  | 19.3 |  | 30.9 |  | 28.8 |  | 25.9 |  | 5 |  |
| BH21-10 | 0.8 | 7.7 | 104 | 0.97 |  | 4.17 |  | 33.3 |  | 378 |  | 137 |  | 33.4 |  | 27.5 |  | 17.9 |  |
| BH22-15 | 0.8 | 7.91 | 50 | 0.39 |  | 1.5 |  | 2.5 |  | 6.8 |  | 19 |  | 8.4 |  | 11 |  | 5.2 |  |
| BH22-16 | 0.8 | 7.83 | 47 | 0.35 |  | 1.3 |  | 2.35 |  | 4 |  | 16 |  | 10 |  | 9.8 |  | 3.6 |  |
| BH22-17 | 0.8 | 7.91 | 54 | 0.45 |  | 1.7 |  | 2.7 |  | 20 |  | 29 |  | 11 |  | 17 |  | 5.1 |  |
| BH22-18 | 0.8 | 8.06 | 47 | 0.33 |  | 2.1 |  | 2.35 |  | 5.4 |  | 23 |  | 7.8 |  | 6.6 |  | 5.3 |  |
| BH22-22 | 0.8 | 7.73 | 71 | 2.5 |  | 4.3 |  | 23 |  | 840 |  | 200 |  | 120 |  | 73 |  | 26 |  |
| Mean |  |  |  |  | 0.6 |  | 2.0 |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard Deviation |  |  |  |  | 0.6 |  | 1.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 95th Percentile |  |  |  |  | 1.5 |  | 4.8 |  |  |  |  |  |  |  |  |  |  |  |  |
| SCARG Category Rating |  |  |  |  | Good |  | Fair |  |  |  |  |  |  |  |  |  |  |  |  |
| Guideline |  |  |  |  | 3 |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |

1.3	Background Deep Rooting Zone Data 1.0 to 1.5 m

| Borehole | Depth | pH | Sat | EC |  | SAR |  | Cl |  | SO4 |  | Na |  | Ca |  | Mg |  | K |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | mbgs | --- | % | dS/m |  | --- |  | mg/kg |  |  |  |  |  |  |  |  |  |  |  |
| BH21-1 | 1.1 | 8.1 | 78 | 0.79 |  | 5.48 |  | 15.3 |  | 143 |  | 101 |  | 2.5 |  | 17.7 |  | 6.1 |  |
| BH21-10 | 1.1 | 7.8 | 98 | 1.97 |  | 3.92 |  | 17.4 |  | 997 |  | 217 |  | 102 |  | 82.6 |  | 34.5 |  |
| BH22-15 | 1.1 | 7.78 | 48 | 5.5 |  | 2.3 |  | 27 |  | 2100 |  | 160 |  | 250 |  | 310 |  | 24 |  |
| BH22-16 | 1.1 | 7.95 | 46 | 0.38 |  | 3.4 |  | 2.3 |  | 12 |  | 29 |  | 4 |  | 4.8 |  | 2.7 |  |
| BH22-17 | 1.1 | 7.86 | 50 | 6.7 |  | 3.2 |  | 56 |  | 2700 |  | 250 |  | 250 |  | 410 |  | 23 |  |
| BH22-18 | 1.1 | 8.05 | 48 | 0.54 |  | 4.7 |  | 2.4 |  | 70 |  | 53 |  | 8.1 |  | 7.2 |  | 4.7 |  |
| BH22-22 | 1.25 | 7.69 | 73 | 3.6 |  | 3.7 |  | 15 |  | 1600 |  | 250 |  | 230 |  | 140 |  | 35 |  |
| BH21-1 | 1.4 | 8.4 | 76 | 2.76 |  | 5.69 |  | 52.3 |  | 996 |  | 276 |  | 16.6 |  | 132 |  | 15 |  |
| BH21-10 | 1.4 | 7.7 | 83 | 1.66 |  | 4.22 |  | 2.5 |  | 669 |  | 171 |  | 57.4 |  | 55.6 |  | 23.5 |  |
| BH22-15 | 1.4 | 7.91 | 48 | 8.3 |  | 3.7 |  | 80 |  | 3200 |  | 330 |  | 270 |  | 610 |  | 24 |  |
| BH22-16 | 1.4 | 8.04 | 44 | 1 |  | 5.2 |  | 40 |  | 97 |  | 79 |  | 17 |  | 13 |  | 3.3 |  |
| BH22-17 | 1.4 | 7.71 | 50 | 8.6 |  | 3.4 |  | 96 |  | 3200 |  | 300 |  | 240 |  | 560 |  | 20 |  |
| BH22-18 | 1.4 | 7.83 | 47 | 5.3 |  | 3.1 |  | 7.1 |  | 2000 |  | 200 |  | 240 |  | 250 |  | 17 |  |
| Mean |  |  |  |  | 3.6 |  | 4.0 |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard Deviation |  |  |  |  | 3.0 |  | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |
| Outlier |  |  |  |  | 9.5 |  | --- |  |  |  |  |  |  |  |  |  |  |  |  |
| 95th Percentile |  |  |  |  | 8.4 |  | 5.6 |  |  |  |  |  |  |  |  |  |  |  |  |
| SCARG Category Rating |  |  |  |  | Poor |  | Fair |  |  |  |  |  |  |  |  |  |  |  |  |
| Guideline |  |  |  |  | 10 |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |

<INSERT_APPE_END>

# Appendix F: Professional Declaration